AeroNavX — Autonomous Drone Project

Power & Propulsion System Engineering Report

Unlox Drone Engineering Program  |  April 2026

# 1. Objective

This report documents the complete power and propulsion system design for the AeroNavX autonomous quadcopter. It covers motor selection, ESC specification, propeller sizing, battery configuration, thrust-to-weight ratio analysis, current draw calculations, and expected flight time under various load conditions.

# 2. Drone Configuration

| Parameter | Value |
| --- | --- |
| Frame Type | Quadcopter X-Configuration |
| Frame Size | 450mm motor-to-motor diagonal |
| All-Up Weight (AUW) | 1.45 kg (including battery, payload, electronics) |
| Target Hover Throttle | 50% (efficient mid-range operation) |
| Target Flight Time | 18–22 minutes |
| Operating Voltage | 14.8V (4S LiPo) |

# 3. Motor Selection

## 3.1 Selected Motor: EMAX 2216 KV880

| Specification | Value |
| --- | --- |
| Motor Model | EMAX 2216 KV880 |
| KV Rating | 880 RPM/V |
| Max Continuous Current | 22A |
| Max Power | 325W |
| Motor Weight | 60g per motor |
| Max Thrust (10x4.5 prop) | 850g per motor |
| Shaft Diameter | 3mm |
| Stator Size | 22mm diameter × 16mm height |

## 3.2 Motor Selection Justification

- KV880 is ideal for 4S battery operation with 10-inch propellers, providing efficient thrust at mid-throttle

- 4 × 850g = 3400g total thrust vs. 1450g AUW gives thrust-to-weight ratio of 2.34:1 (exceeds minimum 2:1 requirement)

- EMAX 2216 is widely used in 450-class racing and aerial photography frames — proven reliability

- Compatible with 30A ESCs for safe operating margin

# 4. ESC (Electronic Speed Controller) Selection

## 4.1 Selected ESC: Hobbywing XRotor 30A

| Specification | Value |
| --- | --- |
| ESC Model | Hobbywing XRotor 30A OPTO |
| Continuous Current | 30A |
| Burst Current (10s) | 40A |
| Battery Input | 2S–4S LiPo (7.4V–16.8V) |
| BEC | None (OPTO type — uses Pixhawk power module) |
| Protocol | PWM / OneShot125 |
| Weight per ESC | 22g |

# 5. Propeller Selection

| Parameter | Value |
| --- | --- |
| Propeller Model | APC 10x4.5 (10 inch, 4.5 pitch) |
| Diameter | 10 inches (254mm) |
| Pitch | 4.5 inches |
| Material | Glass-filled nylon (GFN) |
| Configuration | 2× CW + 2× CCW (counter-rotating pairs) |
| Static Thrust @ 14.8V | 850g per propeller |

# 6. Battery Selection & Sizing

## 6.1 Selected Battery: Tattu 4S 5200mAh LiPo

| Specification | Value |
| --- | --- |
| Model | Tattu 4S 5200mAh 45C LiPo |
| Cell Configuration | 4S1P (4 cells in series) |
| Nominal Voltage | 14.8V (3.7V × 4) |
| Capacity | 5200 mAh |
| Discharge Rating | 45C continuous |
| Max Discharge Current | 45C × 5.2Ah = 234A (far exceeds motor draw) |
| Weight | 380g |

# 7. Thrust-to-Weight Ratio Calculation

| Parameter | Value | Unit |
| --- | --- | --- |
| Single Motor Max Thrust | 850 | grams |
| Total Thrust (4 motors) | 3400 | grams |
| Total AUW | 1450 | grams |
| Thrust-to-Weight Ratio | 2.34 : 1 | — |
| Hover Throttle | ~42.6% | % of max |

Formula: TWR = Total Thrust / AUW = 3400g / 1450g = 2.34

A TWR of 2.34 ensures responsive control, safe payload capacity, and efficient hovering below 50% throttle — the optimal range for extended flight time.

# 8. Current Draw Analysis

| Condition | Current per Motor | Total Current Draw |
| --- | --- | --- |
| Idle / Ground | 1A | 4A |
| Hover (42% throttle) | 6A | 24A |
| Cruise (60% throttle) | 10A | 40A |
| Full Throttle (max) | 22A | 88A |

# 9. Expected Flight Time Calculation

Formula: Flight Time = (Battery Capacity × 0.8) / Average Current Draw × 60 minutes

Note: 0.8 factor applied as safe discharge limit (80% of capacity to protect battery longevity)

| Flight Mode | Avg Current | Estimated Flight Time |
| --- | --- | --- |
| Hover only | 24A | ~10.4 min |
| Mixed Mission (recommended) | 18A | ~13.9 min |
| Gentle Cruise | 14A | ~17.8 min |

Calculation (hover): (5200mAh × 0.8) / 24A = 173.3 min·A / 60 = 10.4 minutes

# 10. Power Distribution Layout

The power distribution follows a hub-and-spoke architecture using a dedicated Power Distribution Board (PDB):

- Battery → XT60 connector → PDB (main power bus)

- PDB → 4× ESC (direct solder pads for minimal resistance)

- PDB → Pixhawk Power Module (5V/2A regulated output)

- Pixhawk → Raspberry Pi (5V via UBEC)

- ESCs → Motors (3-phase brushless connection)

- ESCs → Pixhawk PWM rail (signal only, no power)

# 11. Component Weight Budget

| Component | Quantity | Weight (g) |
| --- | --- | --- |
| F450 Carbon Frame | 1 | 282 |
| EMAX 2216 KV880 Motors | 4 | 240 |
| Hobbywing 30A ESCs | 4 | 88 |
| APC 10x4.5 Propellers | 4 | 60 |
| Tattu 4S 5200mAh LiPo | 1 | 380 |
| Pixhawk 2.4.8 FC | 1 | 38 |
| GPS Module (u-blox M8N) | 1 | 15 |
| Raspberry Pi 4B (companion) | 1 | 46 |
| Camera Module | 1 | 25 |
| Wiring & Misc | — | 76 |
| TOTAL AUW | — | 1250 |

# 12. Conclusion

The AeroNavX propulsion system is designed for reliable autonomous operation with a healthy thrust-to-weight ratio of 2.34:1, ensuring agile control and safe maneuverability. The 4S 5200mAh battery provides sufficient energy for 13–18 minute missions at mixed throttle. The EMAX 2216 KV880 motors with Hobbywing 30A ESCs represent a proven, cost-effective combination for a 450-class quadcopter. All components are commercially available and have been widely validated in similar drone platforms.